Question 1

Complete the following table

| Drop height m | Impact velocity ms-1 | Air time s |
| --- | --- | --- |
| 0.5 |  |  |
| 1 |  |  |
| 1.5 |  |  |
|  |  | 1 |
|  |  | 2 |
|  |  | 3 |
|  | 1 |  |
|  | 2 |  |
|  | 3 |  |

Question 2

A box slides down a roller conveyor. It starts from rest and travels for 3 m. When it reaches the end of the conveyor it is travelling at 4ms-1. Calculate the rate of acceleration.

Question 3

A box is tossed into the air from 1.5 m high. It reaches a height of 3 m before falling to the ground. How long will this take?

Question 4

A pack falls from 1 m hits the ground and rebounds 250 mm. Calculate the coefficient of restitution.

Calculate the velocity change which took place during the impact.

Question 5

A computer specification states that it has a fragility of 50G. What does this mean? How should the packaging engineer use this information in designing distribution packaging.

Question 6

Describe how the damage boundary curve can be used to identify the danger zone for pack damage.

Question 7

Describe the characteristics of the ideal cushion material

Question 8

A laptop weighs 1.5 kg. It is estimated that its fragility factor is 100G. The manufacturer want to protect it from falls of up to 2 m during distribution using expanded polystyrene.

Calculate the thickness and area of cushion required.

Question 10

Discuss the factors which impact on the frequency and servery of drops in distribution.

Question 11

Discuss the factors which impact on the amount of vibration experience in the rear of a truck.

Question 12.

Describe how distribution vibration can be simulated.

Question 13

Describe how vibration can damage products. How can the effect of vibration be mitigated.

Question 14

Discuss the factors which impact on the ability of a corrugated case to protect a product from compression damage.

Question 15

Describe how a packs ability to protect a product during distribution can be evaluated.